Drink and Drive Detection Documentation

Introduction

This project is designed to detect alcohol levels using a gas sensor (MQ), notify users via a Blynk dashboard, and prevent the vehicle from starting if alcohol is detected. It uses an ESP8266 microcontroller for processing and communication with the Blynk IoT platform.

Components Used

Input Layer:

- MQ Sensor: Detects alcohol levels in the environment.

- Digital Switch Sensor: Detects ignition state (ON/OFF).

Processing Layer:

- ESP8266 (NodeMCU): Handles sensor data, decision-making logic, and Blynk communication.

Output Layer:

- Buzzer: Alerts when alcohol is detected.

- Motors (motor_a and motor_b): Simulate vehicle movement or stoppage.

- Blynk Dashboard: Displays sensor data and sends notifications.

Communication Layer:

- Wi-Fi Connectivity: Enables data exchange between the ESP8266 and the Blynk cloud.

Architecture

- Input Layer:

  - The MQ sensor provides analog alcohol readings.

  - A digital pin monitors the ignition switch.

- Processing Layer:

  - The ESP8266 processes sensor inputs and determines the system's state.

  - Implements logic to stop the vehicle if alcohol is detected.

- Output Layer:

  - Activates a buzzer and sends alerts via Blynk.

  - Controls motor signals to stop the vehicle.

- Communication Layer:

  - The system uses Wi-Fi to communicate with the Blynk IoT platform for real-time monitoring and notifications.

System Flow

- Setup Phase:

  - Initialize all components (sensors, motors, and Blynk).

  - Establish Wi-Fi connectivity and authenticate with Blynk using the auth, ssid, and pass credentials.

- Loop Phase:

  - Continuously read sensor data (alcohol level and ignition state).

  - If alcohol is detected:

    - Activate the buzzer.

    - Stop the motor.

    - Notify the user via the Blynk dashboard.

  - If no alcohol is detected:

    - Deactivate the buzzer.

    - Keep the motor running.

  - Send real-time data to the Blynk dashboard using virtual pins.

Libraries Used and Initialization

- ESP8266WiFi.h: Manages Wi-Fi connectivity.

- BlynkSimpleEsp8266.h: Integrates the ESP8266 with the Blynk platform.

Pin Assignments:

- MQ Sensor (Analog): A0

- Buzzer: Pin 4

- Motor Control Pins:

  - Motor A: Pin 12

  - Motor B: Pin 14

- Digital Switch: Pin 13

Wi-Fi and Blynk Credentials:

- Wi-Fi SSID: "vivo Y22"

- Wi-Fi Password: "abcxyz123"

- Blynk Auth Token: "FX4BbDNa1F8QFiN5vh_g3Ae0JiHOWbX9"

Guide to Using Blynk

1. Sign Up and Login:

- Download the Blynk IoT app from Google Play or the App Store.

- Create an account or log in to an existing one.

2. Create a New Template:

- Navigate to Templates in the app or Blynk web dashboard.

- Create a new template with the desired name (e.g., "Drink and Drive Detection").

- Note the Template Name, Template ID, and Auth Token generated.

3. Add a New Device:

- Go to the Devices section and add a device under the created template.

- Copy the device-specific Auth Token.

4. Configure Data Streams:

- Define Virtual Pins (e.g., V0 for LED, V1 for sensor data) in the Datastreams tab.

- Specify the data type (integer/float) and value ranges.

5. Design the Dashboard:

- Add widgets to the dashboard:

  - LED Widget linked to V0.

  - Value Display widget linked to V1 for sensor values.

  - Notification Widget for alerts.

Configuring Wi-Fi and Blynk in Code

- Replace the placeholders with your credentials:

- #define WIFI_SSID "Your WiFi SSID"

- #define WIFI_PASSWORD "Your WiFi Password"

- #define BLYNK_AUTH_TOKEN "Your Device Auth Token"

- Ensure the code has the correct virtual pins defined for data communication:

#define virtual_led V0

Conclusion

The Drink and Drive Detection System integrates IoT and real-time monitoring to enhance safety by preventing driving under the influence of alcohol. It combines sensor technology, microcontroller processing, and the Blynk platform to create a comprehensive safety solution. This project ensures early detection and user alerts to mitigate risks effectively.