Enhanced Medical Data Privacy and Security

Introduction

This system integrates IoT and cloud technologies to securely manage medical data using RFID and Firebase. It ensures data security, user authentication, and seamless interaction through a Liquid Crystal Display (LCD) and a Flutter app interface. The architecture is designed to prioritize privacy, efficiency, and real-time data accessibility.

System Architecture

1. Input Layer

- RFID Module (MFRC522): Scans and reads unique IDs from RFID cards to identify users.

- Push Button: Provides a tactile interface for user actions like confirming or progressing through the displayed data.

2. Processing Layer

- ESP32/ESP8266 Microcontroller:

  - Handles the flow of data between the RFID module, Firebase, and LCD.

  - Authenticates with Firebase using API credentials and manages database operations.

  - Implements scrolling functionality for lengthy data fields.

- Firebase Realtime Database:

  - Serves as the secure storage for user details.

  - Processes database queries to retrieve and update user records.

3. Communication Layer

- Wi-Fi (ESP32/ESP8266):

  - Establishes a wireless connection to enable real-time data exchange between the microcontroller and Firebase.

- Firebase Token Management:

  - Ensures encrypted and secure communication with the Firebase Realtime Database.

4. Output Layer

- Liquid Crystal Display (LCD):

  - Displays user data, including UID, name, age, and description.

  - Implements a scrolling feature for displaying lengthy information.

- Flutter App Interface:

  - Provides a remote view and management of medical data stored in Firebase.

Components Used

- RFID Module (MFRC522): Reads unique IDs from RFID cards for user authentication.

- ESP32/ESP8266 Microcontroller: Manages component integration, Wi-Fi connectivity, and data processing.

- Liquid Crystal Display (LCD): Outputs user details and provides a user-friendly interface.

- Push Button: Facilitates user interactions for navigating displayed data.

- Firebase: Secure cloud-based storage for medical data.

- Flutter App: Acts as a remote management and monitoring tool for medical records.

Libraries Used and Initialization

Libraries:

- SPI.h, MFRC522.h: Interface with the RFID module.

- Firebase_ESP_Client.h: Handles Firebase operations and secure token management.

- LiquidCrystal_I2C.h: Manages LCD display operations.

- WiFi.h/ESP8266WiFi.h: Enables Wi-Fi connectivity for the ESP32/ESP8266 boards.

Initialization:

- Wi-Fi Configuration:

- #define WIFI_SSID "saurabh"

- #define WIFI_PASSWORD "12345687"

- Firebase Configuration:

  - #define API_KEY "AIzaSyCcxY7Ju6SE8pwzehalawpJCCnwZ4S3ToQ"

  - #define DATABASE_URL "https://medical-f4bff-default-rtdb.firebaseio.com/"

- Pin Assignments:

  - SS_PIN (4): RFID SDA pin.

  - RST_PIN (15): RFID RST pin.

  - BTN_PIN (5): Push button pin.

  - LED_PIN (27): Reserved for visual feedback.

System Flow

1. Setup Phase

- Wi-Fi Connection: Establishes connection to the Wi-Fi network and logs the IP address.

- Firebase Initialization: Authenticates the system with Firebase and prepares for database operations.

- LCD and RFID Setup: Initializes the LCD for data display and configures the RFID module for scanning cards.

2. Main Loop

- Card Detection:

  - Continuously scans for RFID cards.

  - When a card is detected, its UID is displayed on the LCD and serial monitor.

- Data Retrieval:

  - Constructs the database path using the detected UID.

  - Fetches user information (e.g., name, age, and medical description) from Firebase.

  - Displays retrieved information on the LCD and logs it in the serial monitor.

- User Interaction:

  - The push button allows the user to scroll through displayed data fields sequentially.

- Error Handling:

  - Displays error messages if the UID is not found in Firebase.

- Scrollable Data:

  - Implements a scrolling mechanism for lengthy data fields (e.g., medical descriptions).

Integration with Flutter App

1. User Interface:

- Displays medical data in a structured format for authorized users.

- Supports adding, updating, and deleting records in Firebase.

2. Synchronization:

- Real-time updates between Firebase and the microcontroller.

- Ensures data consistency across all access points.

Key Features

- Secure Data Management: Encrypted communication with Firebase ensures data privacy.

- User-Friendly Interface: LCD provides an intuitive way to display user information.

- Remote Management: Flutter app allows real-time monitoring and updates.

This documentation captures the complete flow and technical aspects of Enhanced Medical Data Privacy and Security, providing insights into its architecture and functionality.