Greenhouse Automation Documentation

Introduction

This system automates and remotely controls the operations of a greenhouse. Using Blynk, ESP8266, and sensors such as DHT11 for temperature and humidity, as well as a moisture sensor for soil, the system adjusts the fan, pump, and monitors environmental conditions. The user can control the system via the Blynk mobile app, which shows real-time data and allows manual control of devices.

Components Used

- DHT11 Sensor: Measures temperature and humidity in the greenhouse.

- Soil Moisture Sensor (A0): Monitors soil moisture to determine when to activate the pump.

- ESP8266 Microcontroller: Manages all components, handles Wi-Fi connection, and communicates with the Blynk app.

- Fan Motors: Controlled to manage temperature by turning the fan on or off.

- Water Pump Motors: Controlled to water the plants when required.

- Blynk Mobile App: Provides remote control and monitoring via a dashboard.

- Wi-Fi: Used for internet connectivity between the ESP8266 and Blynk app.

Libraries Used and Initialization

Libraries:

- DHT.h: Used to interface with the DHT11 sensor for temperature and humidity readings.

- ESP8266WiFi.h: Manages the Wi-Fi connection between the microcontroller and Blynk.

- BlynkSimpleEsp8266.h: Manages the interaction with Blynk, providing data streaming and remote control features.

Initialization:

- Wi-Fi Credentials:

- #define WIFI_SSID "vivo Y22"

- #define WIFI_PASSWORD "abcxyz123"

Connects the microcontroller to the defined Wi-Fi network.

- Blynk Credentials:

  - Auth Token: char auth[] = "2xH4CgG6pBn3nBXJtoeBGGYUUXIX_wX2";

  - Blynk Template ID and Name:

  - #define BLYNK_TEMPLATE_ID "TMPL3JEuka6Oc"

  - #define BLYNK_TEMPLATE_NAME "Green House Automation"

The auth token and template_id are used for connecting the device to the Blynk app.

- Pin Assignments:

  - DHT11_PIN (4): Pin for DHT11 sensor.

  - AOUT_PIN (A0): Analog pin for soil moisture sensor.

  - fanv1 (12), fanv2 (14): Pins controlling the fan motors.

  - motor1 (15), motor2 (13): Pins controlling the water pump motors.

System Architecture

Input Layer:

- DHT11 Sensor: Measures the temperature and humidity of the greenhouse.

- Soil Moisture Sensor: Detects moisture levels in the soil, triggering the pump when necessary.

Output Layer:

- Fan Motors: Turned on or off based on the temperature in the greenhouse.

- Water Pump Motors: Turned on or off based on soil moisture or manual control via the Blynk app.

Communication:

- ESP8266 Wi-Fi Module: Communicates data between the sensors, output devices, and the Blynk app using the internet.

Processing Layer:

- ESP8266 Microcontroller: Processes the data from sensors and sends it to the Blynk app. It also processes commands from the app and controls the connected devices (fan and pump).

System Flow

Setup Phase:

- Wi-Fi Connection: The ESP8266 connects to the specified Wi-Fi network.

- Blynk Initialization: The ESP8266 connects to the Blynk cloud using the provided authentication token.

- Pin Initialization: Initializes the required pins for the sensors, fan, and pump motors.

Loop Phase:

- Reading Sensor Data: The system reads temperature, humidity, and soil moisture data.

- Fan Control: If the temperature exceeds a certain threshold (29°C), the fan is turned on. Otherwise, it remains off.

- Pump Control: If the moisture level is below a set threshold or manual control is enabled via the Blynk app, the pump is turned on.

- Data Display on Blynk: The temperature, humidity, and moisture levels are sent to the Blynk app for real-time monitoring.

Integration with Blynk App

Getting Started with Blynk:

- Create a Blynk Account:

  - Download the Blynk app from the App Store or Google Play.

  - Open the app and sign up with your email or use an existing social login.

- Create a New Project in Blynk:

  - Open the Blynk app and create a new project by clicking the + button.

  - Name your project (e.g., "Greenhouse Automation").

  - Select the hardware as ESP8266 and choose your connection type as Wi-Fi.

- Get the Auth Token:

  - After creating the project, Blynk will generate an Auth Token.

  - Copy the Auth Token, as you’ll need it for the auth[] variable in the code.

- Create the Template:

  - Inside the Blynk app, go to Templates and create a new template for your project. You can name it Green House Automation.

  - Note down the Template ID and Template Name. These will be used in the code for authentication.

- Adding Widgets:

  - Add Value Display widgets to show real-time data for temperature (V1), humidity (V2), and moisture (V3).

  - Add Button widgets for controlling the pump (V4) and fan (V5).

  - Configure each widget with the corresponding Virtual Pins (V1, V2, V3, etc.) defined in the code.

- Set Up Data Streams:

  - Define Data Streams for each widget (e.g., temperature, humidity, moisture levels) to update regularly.

  - Set the frequency of data updates to match the system's requirements.

- Set Up Controls:

  - Configure the Button widgets to send commands to control the pump and fan based on user input from the app.

Connecting to Wi-Fi and Blynk:

- Wi-Fi Credentials:

  - Provide your Wi-Fi credentials in the code as follows:

  - #define WIFI_SSID "your_wifi_name"

  - #define WIFI_PASSWORD "your_wifi_password"

  - This allows the ESP8266 to connect to your local Wi-Fi network.

- Blynk Credentials:

  - Use the Auth Token and Template ID/Name provided by the Blynk app:

  - char auth[] = "your_blynk_auth_token";

  - #define BLYNK_TEMPLATE_ID "your_template_id"

  - #define BLYNK_TEMPLATE_NAME "your_template_name"

Conclusion

The Greenhouse Automation project integrates IoT with Blynk to create an automated system for managing environmental conditions within a greenhouse. The system reads temperature, humidity, and soil moisture, and automatically controls the fan and water pump based on these readings. Users can remotely monitor and control the greenhouse environment using the Blynk mobile app. The system ensures optimal conditions for plant growth while being flexible enough for manual control.