IoT Industry Protection System Architecture

1. Overview

The system uses an ESP32 microcontroller for sensor data processing, Blynk integration, and sending control signals to an Arduino. The Arduino handles the relays for the motor (water pump) and fan, ensuring seamless control and monitoring. Sensors including the DHT11, MQ3, Flame Sensor, and PIR Sensor gather environmental data, while the actuators (motor and fan) are controlled via relays based on the sensor readings. The Blynk IoT platform is used for remote monitoring, control, and event-driven alerts.

2.  Hardware Components

2.1 Microcontroller Setup

- ESP32:

  - Collects sensor data and processes it.

  - Sends control signals to Arduino for relay management (motor and fan).

  - Integrates with the Blynk platform for remote monitoring and control.

- Arduino:

  - Receives control signals from the ESP32 via GPIO pins.

  - Controls relays for the fan and motor based on the received signals.

2.2 Relays

- Relay for Motor: Connected to Arduino Pin 7.

- Relay for Fan: Connected to Arduino Pin 8.

2.3 Communication Between ESP32 and Arduino

- ESP32 Output Pins:

  - motor_control_pin (2): Sends signal for motor control.

  - fan_control_pin (4): Sends signal for fan control.

- Arduino Input Pins:

  - motor_input (2): Reads motor control signal.

  - fan_input (3): Reads fan control signal.

2.4 Sensors

- DHT11: Measures temperature and humidity.

- MQ3: Detects gas leakage.

- Flame Sensor: Detects flames.

- PIR Sensor: Detects motion.

2.5 Actuators

- Motor (Water Pump): Controlled via relay connected to Arduino pin 7.

- Fan (Cooling System): Controlled via relay connected to Arduino pin 8.

3. Communication Flow

3.1 ESP32 to Arduino Communication

- ESP32 Logic:

  - Determines whether to turn the motor and fan on or off based on sensor readings.

  - Sends digital HIGH (1) or LOW (0) signals to Arduino pins:

    - Pin 2 for motor control.

    - Pin 4 for fan control.

- Arduino Logic:

  - Reads signals from ESP32 pins (motor_input, fan_input).

  - Activates or deactivates relays for motor and fan based on received signals.

4. Software Integration

4.1 ESP32 Code Logic

- Detects high temperature, flame, or gas leakage.

- Sets control pins to HIGH or LOW based on sensor readings.

- Sends control signals to Arduino for relay management.

4.2 Arduino Code Logic

- Reads input from ESP32 signals (motor_input, fan_input).

- Activates or deactivates relays for motor and fan.

- Ensures relays are in a default HIGH state (OFF) when no signal is received or the input is LOW.

5. Event Alerts and Logging

To implement event-driven alerts using the Blynk platform, the system will notify the user when specific thresholds are crossed. The system uses the Blynk app to log events and send notifications for detected hazards:

- Gas Leakage: If the MQ3 sensor detects high gas levels, an event will be triggered.

Blynk.logEvent("gas_leakage", "Gas detected in the environment");

- Flame Detection: If the Flame Sensor detects a flame, the system will log the event.

Blynk.logEvent("flame_detected", "Flame detected in the environment");

6. Code Workflow

6.1 ESP32

- Initialization:

  - Sets up WiFi and Blynk connection.

  - Configures sensors and control pins (motor_control_pin, fan_control_pin).

- Main Loop:

  - Reads sensor data (temperature, humidity, gas, flame, motion).

  - Sends HIGH or LOW signals to control pins based on logic.

6.2 Arduino

- Initialization:

  - Configures input pins for receiving signals from ESP32.

  - Configures output pins for controlling relays (motorRelay, fanRelay).

- Main Loop:

  - Reads input signals from ESP32.

  - Activates or deactivates relays for fan and motor.

7. Functional Workflow

Normal Operation

- ESP32:

  - Monitors sensor data.

  - Sends control signals to Arduino for motor and fan based on readings.

- Arduino:

  - Activates or deactivates relays for motor and fan based on ESP32 signals.

Emergency Handling

- Fan activates for high temperature.

- Motor activates for flame detection.

- Buzzer activates for gas or motion detection.

Manual Control

- Users can control the fan and motor manually via the Blynk app.

- ESP32 sends control signals to Arduino for relay management.

8. Advantages of Adding Arduino

- Offloading Relay Control: Arduino simplifies ESP32 processing by handling relay control.

- Isolation: Ensures separate control for sensors (ESP32) and actuators (Arduino).

- Modularity: Simplifies debugging and system maintenance.

9. How to Add Blynk App for Remote Control

Step 1: Install the Blynk App

- Download the Blynk IoT app from:

  - Google Play Store (for Android)

  - Apple App Store (for iOS).

- Create a Blynk account using your email or log in if you already have one.

Step 2: Create a New Template

- Visit the Blynk Console (web dashboard).

- Click on Templates > + New Template.

- Fill in:

  - Name: IoT Industry Protection.

  - Hardware: ESP32.

  - Connection Type: WiFi.

- Click Create.

- Copy the Template ID, Template Name, and Auth Token for use in the ESP32 code.

Step 3: Add Data Streams

- In the Blynk Console, open your template.

- Click Datastreams > + New Datastream.

- Add the following data streams:

| Datastream Name | Virtual Pin | Data Type | Min/Max Value | Purpose |
| --- | --- | --- | --- | --- |
| Temperature | V1 | Float | 0-100 | DHT11 Temperature data. |
| Humidity | V2 | Float | 0-100 | DHT11 Humidity data. |
| Temperature LED | V3 | Integer | 0-1 | High Temp indicator (LED). |
| Flame LED | V4 | Integer | 0-1 | Flame detection indicator. |
| Gas LED | V5 | Integer | 0-1 | Gas detection indicator. |
| Motion LED | V6 | Integer | 0-1 | PIR motion detection indicator. |
| Fan Control | V7 | Integer | 0-1 | Control fan relay. |
| Motor Control | V8 | Integer | 0-1 | Control motor relay. |

- Save the data streams.

Step 4: Configure the Blynk Mobile Dashboard

- In the Blynk app, add a new device using the template you created.

- Add the following widgets:

  - Label (Temperature): Displays temperature.

  - Label (Humidity): Displays humidity.

  - LED (Temp Alert): Turns ON for high temperatures.

  - LED (Flame Alert): Turns ON for flame detection.

  - LED (Gas Alert): Turns ON for gas detection.

  - LED (Motion Alert): Turns ON for motion detection.

  - Button (Fan Control): Manually control the fan.

  - Button (Motor Control): Manually control the motor.

- Customize the widgets to link to the respective virtual pins.

- Save and sync the dashboard.

Step 5: Connect and Test

- Power on your ESP32 and Arduino setup.

- Open the Blynk app to monitor sensor values in real-time and test manual controls for the fan and motor.

Key Notes:

- Ensure a stable WiFi connection.

- Add your WiFi credentials (SSID and password) to the ESP32 code.

- Copy and paste the Template ID, Template Name, and Auth Token into the ESP32 code.