System Architecture for IoT Industry Protection

1. Overview

The system uses an ESP32 to handle sensor data processing, Blynk integration, and control signals. The Arduino receives control signals from the ESP32 via specific pins and controls the relays for the motor (water pump) and fan.

2. Updated Hardware Components

2.1 Microcontroller Setup

- ESP32:

  - Collects sensor data and processes it.

  - Sends control signals to the Arduino through GPIO pins.

  - Integrates with Blynk IoT for remote monitoring and control.

- Arduino:

  - Receives digital input from the ESP32.

  - Controls relays for the fan and motor based on the received signals.

2.2 Relays

- Relay for Motor: Connected to Arduino Pin 7.

- Relay for Fan: Connected to Arduino Pin 8.

2.3 Communication Between ESP32 and Arduino

- ESP32 Output Pins:

  - motor_control_pin (2): Sends control signal for the motor.

  - fan_control_pin (4): Sends control signal for the fan.

- Arduino Input Pins:

  - motor_input (2): Reads the motor control signal.

  - fan_input (3): Reads the fan control signal.

2.4 Sensors

Same as before (DHT11, MQ3, Flame Sensor, PIR Sensor).

2.5 Actuators

- Motor (Water Pump): Controlled via relay connected to Arduino pin 7.

- Fan (Cooling System): Controlled via relay connected to Arduino pin 8.

3. Communication Flow

3.1 ESP32 to Arduino Communication

- ESP32 Logic:

  - Determines the ON/OFF state for the motor and fan based on sensor readings.

  - Sends digital HIGH (1) or LOW (0) signals to the Arduino pins:

    - Pin 2 for motor control.

    - Pin 4 for fan control.

- Arduino Logic:

  - Reads digital signals on pins 2 (motor_input) and 3 (fan_input).

  - Activates or deactivates relays connected to pins 7 (motorRelay) and 8 (fanRelay) based on the received signals.

4. Software Integration

4.1 ESP32 Code Logic

- Detects high temperature, flame, or other conditions.

- Sets control pins (motor_control_pin, fan_control_pin) to HIGH or LOW based on sensor data.

- Sends these signals to the Arduino.

4.2 Arduino Code Logic

- Reads input from ESP32 pins (motor_input, fan_input).

- Activates or deactivates the motor and fan relays accordingly.

- Ensures default HIGH state (relays OFF) when no signal is received or the input is LOW.

6. Code Workflow

6.1 ESP32

- Initialization:

  - Sets up WiFi and Blynk connection.

  - Configures sensors and control pins (motor_control_pin, fan_control_pin).

- Main Loop:

  - Reads sensor data.

  - Processes logic for activating fan and motor.

  - Sends HIGH or LOW signals to control pins based on logic.

6.2 Arduino

- Initialization:

  - Configures input pins for ESP32 signals (motor_input, fan_input).

  - Configures output pins for relays (motorRelay, fanRelay).

- Main Loop:

  - Reads input signals from ESP32.

  - Activates or deactivates relays for the fan and motor.

  - Logs relay status to the Serial Monitor.

7. Functional Workflow

Normal Operation

- ESP32:

  - Monitors sensor data and evaluates conditions.

  - Sends appropriate control signals to the Arduino.

- Arduino:

  - Activates or deactivates relays for the fan and motor based on ESP32 signals.

Emergency Handling

- Same as previous architecture:

  - Fan activates for high temperature.

  - Motor activates for flame detection.

  - Buzzer activates for gas or motion detection.

Manual Control

- Users can control the fan and motor via Blynk app buttons.

- ESP32 sends the respective signals to the Arduino for relay control.

8. Advantages of Adding Arduino

- Offloading Relay Control: Arduino handles the relays, simplifying ESP32 processing.

- Isolation: Ensures separate control layers for sensors (ESP32) and actuators (Arduino).

- Modularity: Allows easier debugging and maintenance.

How to add blynk app

Step 1: Install the Blynk App

- Download the Blynk IoT app from:

  - Google Play Store (for Android)

  - Apple App Store (for iOS).

- Create a Blynk account using your email or log in if you already have one.

Step 2: Create a New Template

- Go to the Web Dashboard:

  - Visit Blynk Console.

- Create a Template:

  - Click on Templates > + New Template.

  - Fill in:

    - Name: IoT Industry Protection.

    - Hardware: ESP32.

    - Connection Type: WiFi.

  - Click Create.

- Note Template Details:

  - Copy:

    - Template ID

    - Template Name

    - Auth Token

Step 3: Add Data Streams

- In the Blynk Console, open your template.

- Click Datastreams in the left menu and then + New Datastream.

- Add data streams for your sensors and controls as follows:

| Datastream Name | Virtual Pin | Data Type | Min/Max Value | Purpose |
| --- | --- | --- | --- | --- |
| Temperature | V1 | Float | 0-100 | DHT11 Temperature data. |
| Humidity | V2 | Float | 0-100 | DHT11 Humidity data. |
| Temperature LED | V3 | Integer | 0-1 | High Temp indicator (LED). |
| Flame LED | V4 | Integer | 0-1 | Flame detection indicator. |
| Gas LED | V5 | Integer | 0-1 | Gas detection indicator. |
| Motion LED | V6 | Integer | 0-1 | PIR motion detection indicator. |
| Fan Control | V7 | Integer | 0-1 | Control fan relay. |
| Motor Control | V8 | Integer | 0-1 | Control motor relay. |

- Save the Data Streams:

  - After configuring each stream, click Create or Save.

Step 4: Configure the Blynk Mobile Dashboard

- Add a New Device:

  - In the Blynk app, click the + Add New Device button.

  - Choose Template and select the one you just created.

- Add Widgets:

  - Go to the dashboard and add the following widgets:

| Widget | Virtual Pin | Purpose |
| --- | --- | --- |
| Label (Temperature) | V1 | Display temperature (°C). |
| Label (Humidity) | V2 | Display humidity (%). |
| LED (Temp Alert) | V3 | Turns ON if temperature > 30°C. |
| LED (Flame Alert) | V4 | Turns ON if flame is detected. |
| LED (Gas Alert) | V5 | Turns ON if gas is detected. |
| LED (Motion Alert) | V6 | Turns ON if motion is detected. |
| Button (Fan Control) | V7 | Manually control the fan. |
| Button (Motor Control) | V8 | Manually control the motor. |

- Customize Widgets:

  - Label Widgets:

    - Set labels like "Temperature: [value] °C" or "Humidity: [value] %".

  - LED Widgets:

    - Link each LED to its respective virtual pin.

    - Set ON color to red/green as desired.

  - Button Widgets:

    - Set Mode to "Switch".

    - Link Fan Button to V7 and Motor Button to V8.

- Save and Sync:

  - Save the dashboard, and it will automatically sync with your device.

Step 5: Connect and Test

- Power On your ESP32 and Arduino setup.

- Open the Blynk App:

  - Monitor sensor values in real-time on the app dashboard.

  - Test manual controls for the fan and motor using the buttons.

- Verify Actuators:

  - Check if:

    - LEDs turn ON/OFF for respective alerts.

    - Fan and motor relays are triggered via manual control.

Key Notes

- Ensure a stable WiFi connection.

- Ensure a you have added you WiFi name and pass to esp code and upload it.

- Template ID ,Template Name,  Auth Token that you copy paste it to a esp code.